Name: Mohamed Ahmed Mohamed Hussein   240102962

ESP32-CAM

Ai Thinker ESP32 CAM Development Board WiFi+Bluetooth with OV2640 Camera Module

The code that makes it works alone :

#include "esp_camera.h"

#include <WiFi.h>

#include <WebServer.h>

// ========= WIFI =========

const char* ssid = "AHMED";

const char* password = "KLM1375H";

// ========= CAMERA (ESP32 WROVER + OV2640) =========

#define CAMERA_MODEL_WROVER_KIT

#if defined(CAMERA_MODEL_WROVER_KIT)

#define PWDN_GPIO_NUM    -1

#define RESET_GPIO_NUM   -1

#define XCLK_GPIO_NUM     21

#define SIOD_GPIO_NUM     26

#define SIOC_GPIO_NUM     27

#define Y9_GPIO_NUM       35

#define Y8_GPIO_NUM       34

#define Y7_GPIO_NUM       39

#define Y6_GPIO_NUM       36

#define Y5_GPIO_NUM       19

#define Y4_GPIO_NUM       18

#define Y3_GPIO_NUM        5

#define Y2_GPIO_NUM        4

#define VSYNC_GPIO_NUM    25

#define HREF_GPIO_NUM     23

#define PCLK_GPIO_NUM     22

#endif

WebServer server(80);

void handle_jpg_stream() {

WiFiClient client = server.client();

camera_fb_t * fb = esp_camera_fb_get();

if(!fb) {

Serial.println("Camera capture failed!");

return;

}

client.write("HTTP/1.1 200 OK\r\n");

client.write("Content-Type: image/jpeg\r\n\r\n");

client.write(fb->buf, fb->len);

esp_camera_fb_return(fb);

}

void setup() {

Serial.begin(115200);

Serial.println();

// CAMERA CONFIG

camera_config_t config;

config.ledc_channel = LEDC_CHANNEL_0;

config.ledc_timer   = LEDC_TIMER_0;

config.pin_d0 = Y2_GPIO_NUM;

config.pin_d1 = Y3_GPIO_NUM;

config.pin_d2 = Y4_GPIO_NUM;

config.pin_d3 = Y5_GPIO_NUM;

config.pin_d4 = Y6_GPIO_NUM;

config.pin_d5 = Y7_GPIO_NUM;

config.pin_d6 = Y8_GPIO_NUM;

config.pin_d7 = Y9_GPIO_NUM;

config.pin_xclk = XCLK_GPIO_NUM;

config.pin_pclk = PCLK_GPIO_NUM;

config.pin_vsync = VSYNC_GPIO_NUM;

config.pin_href = HREF_GPIO_NUM;

config.pin_sscb_sda = SIOD_GPIO_NUM;

config.pin_sscb_scl = SIOC_GPIO_NUM;

config.pin_pwdn = PWDN_GPIO_NUM;

config.pin_reset = RESET_GPIO_NUM;

config.xclk_freq_hz = 20000000;

config.pixel_format = PIXFORMAT_JPEG;

config.frame_size   = FRAMESIZE_QVGA;   // Safer for testing

config.jpeg_quality = 12;

config.fb_count     = 1;

if (esp_camera_init(&config) != ESP_OK) {

Serial.println("❌ Camera init failed!");

return;

}

Serial.println("📷 Camera OK");

// WIFI CONNECT

WiFi.begin(ssid, password);

Serial.print("Connecting");

while (WiFi.status() != WL_CONNECTED) {

Serial.print(".");

delay(500);

}

Serial.println("\n✔️ WiFi connected");

Serial.print("OPEN in browser ➤ http://");

Serial.println(WiFi.localIP());

// Start server

server.on("/", handle_jpg_stream);

server.begin();

}

void loop() {

server.handleClient();

}

Photo with camera after applying the code :

The IP address can be entered in any browser to view the images, which are updated at regular intervals you decided on code .

The code with server :

#include "esp_camera.h"

#include <WiFi.h>

#include <HTTPClient.h>

// ===== WIFI =====

const char* ssid = "AHMED";

const char* password = "KLM1375H";

// ===== NGROK UPLOAD URL =====

// MUST end with /upload

const char* uploadUrl = "https://true-crabs-march.loca.lt/upload";

// ===== CAMERA PINS (ESP32 WROVER KIT) =====

#define PWDN_GPIO_NUM     -1

#define RESET_GPIO_NUM    -1

#define XCLK_GPIO_NUM      21

#define SIOD_GPIO_NUM      26

#define SIOC_GPIO_NUM      27

#define Y9_GPIO_NUM        35

#define Y8_GPIO_NUM        34

#define Y7_GPIO_NUM        39

#define Y6_GPIO_NUM        36

#define Y5_GPIO_NUM        19

#define Y4_GPIO_NUM        18

#define Y3_GPIO_NUM         5

#define Y2_GPIO_NUM         4

#define VSYNC_GPIO_NUM     25

#define HREF_GPIO_NUM      23

#define PCLK_GPIO_NUM      22

void setup() {

Serial.begin(115200);

Serial.println("Booting...");

// ===== CAMERA CONFIG =====

camera_config_t config;

config.ledc_channel = LEDC_CHANNEL_0;

config.ledc_timer = LEDC_TIMER_0;

config.pin_d0 = Y2_GPIO_NUM;

config.pin_d1 = Y3_GPIO_NUM;

config.pin_d2 = Y4_GPIO_NUM;

config.pin_d3 = Y5_GPIO_NUM;

config.pin_d4 = Y6_GPIO_NUM;

config.pin_d5 = Y7_GPIO_NUM;

config.pin_d6 = Y8_GPIO_NUM;

config.pin_d7 = Y9_GPIO_NUM;

config.pin_xclk = XCLK_GPIO_NUM;

config.pin_pclk = PCLK_GPIO_NUM;

config.pin_vsync = VSYNC_GPIO_NUM;

config.pin_href = HREF_GPIO_NUM;

config.pin_sscb_sda = SIOD_GPIO_NUM;

config.pin_sscb_scl = SIOC_GPIO_NUM;

config.pin_pwdn = PWDN_GPIO_NUM;

config.pin_reset = RESET_GPIO_NUM;

config.xclk_freq_hz = 20000000;

config.pixel_format = PIXFORMAT_JPEG;

config.frame_size = FRAMESIZE_QVGA;

config.jpeg_quality = 12;

config.fb_count = 1;

if (esp_camera_init(&config) != ESP_OK) {

Serial.println("❌ Camera init failed!");

return;

}

Serial.println("📷 Camera started.");

// ===== WIFI =====

WiFi.begin(ssid, password);

Serial.print("Connecting to WiFi");

while (WiFi.status() != WL_CONNECTED) {

Serial.print(".");

delay(400);

}

Serial.println("\n✔ WiFi connected");

}

void loop() {

Serial.println("📸 Capturing image...");

camera_fb_t *fb = esp_camera_fb_get();

if (!fb) {

Serial.println("❌ Capture failed");

delay(2000);

return;

}

Serial.printf("Image size: %d bytes\n", fb->len);

// ===== BYPASS SSL HANDSHAKE =====

WiFiClientSecure client;

client.setInsecure();    // <<< THIS BYPASSES CERTIFICATE VALIDATION

HTTPClient http;

http.begin(client, uploadUrl);

// =========================================================

// === AUTHENTICATION HEADER ADDED HERE ====================

// =========================================================

// Base64-encoded value for credentials: :156.202.45.89

const char* authHeaderValue = "Basic OjE1Ni4yMDIuNDUuODk=";

http.addHeader("Authorization", authHeaderValue);

// =========================================================

http.addHeader("Content-Type", "image/jpeg");

// Send POST request

int response = http.POST(fb->buf, fb->len);

Serial.print("Server Response: ");

Serial.println(response);

if (response > 0) {

Serial.println(http.getString());

} else {

Serial.println("❌ Failed to send POST");

}

http.end();

esp_camera_fb_return(fb);

delay(5000); // wait 5 seconds before next photo

}

This indicate that the photo that had captured send to server by the Tunnel URL on the code .

From the server other point of view .

#define TRIG_PIN D5  // GPIO14

#define ECHO_PIN D6  // GPIO12

void setup() {

Serial.begin(115200);

pinMode(TRIG_PIN, OUTPUT);

pinMode(ECHO_PIN, INPUT);

}

void loop() {

long duration;

float distance_cm;

// Send a 10µs pulse to trigger

digitalWrite(TRIG_PIN, LOW);

delayMicroseconds(2);

digitalWrite(TRIG_PIN, HIGH);

delayMicroseconds(10);

digitalWrite(TRIG_PIN, LOW);

// Read the echo pulse duration

duration = pulseIn(ECHO_PIN, HIGH);

// Calculate distance in cm

distance_cm = (duration * 0.0343) / 2;

Serial.print("Distance: ");

Serial.print(distance_cm);

Serial.println(" cm");

delay(500);

}

The GPS NEO-6M GPS Module with EPROM

#include <SoftwareSerial.h>

#include <TinyGPSPlus.h>

#include <ESP8266WiFi.h>

// ------------------

// GPS Setup

// ------------------

const int RXPin = 4; // D2

const int TXPin = 5; // D1

SoftwareSerial neo6m(RXPin, TXPin);

TinyGPSPlus gps;

// ------------------

// WiFi Setup

// ------------------

const char* ssid = "AHMED";

const char* password = "KLM1375H";

WiFiServer server(80);

// ------------------

// Last known location

// ------------------

double lastLat = 0.0;

double lastLng = 0.0;

void setup() {

Serial.begin(115200);

delay(100);

neo6m.begin(9600);

// Connect to WiFi

Serial.print("Connecting to WiFi: ");

Serial.println(ssid);

WiFi.begin(ssid, password);

int retries = 0;

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

retries++;

if (retries > 30) {

Serial.println("\nFailed to connect. Restarting...");

ESP.restart();

}

}

Serial.println("\nWiFi connected!");

Serial.print("IP Address: ");

Serial.println(WiFi.localIP());

server.begin();

}

void loop() {

// Read GPS data

while (neo6m.available()) {

gps.encode(neo6m.read());

}

// Update last known coordinates if GPS fix available

if (gps.location.isValid()) {

lastLat = gps.location.lat();

lastLng = gps.location.lng();

Serial.print("Latitude: "); Serial.println(lastLat, 6);

Serial.print("Longitude: "); Serial.println(lastLng, 6);

}

// Handle web client

WiFiClient client = server.available();

if (client) {

Serial.println("New client connected");

String currentLine = "";

while (client.connected()) {

if (client.available()) {

char c = client.read();

if (c == '\n') {

if (currentLine.length() == 0) {

// Build HTML

String html = "<!DOCTYPE html><html><head>";

html += "<meta charset='UTF-8'><meta name='viewport' content='width=device-width, initial-scale=1.0'>";

html += "<title>ESP8266 GPS Tracker</title></head><body>";

html += "<h1>ESP8266 GPS Tracker</h1>";

if (gps.location.isValid()) {

html += "<p>Latitude: " + String(lastLat,6) + "</p>";

html += "<p>Longitude: " + String(lastLng,6) + "</p>";

html += "<p>GPS Fix Acquired!</p>";

} else {

html += "<p>Searching for satellites...</p>";

}

html += "</body></html>";

client.print(html);

break;

} else {

currentLine = "";

}

} else if (c != '\r') {

currentLine += c;

}

}

yield();

}

client.stop();

Serial.println("Client disconnected");

}

}

The component that we should buy

4* Ultrasonic sensors for the classification portions  80*4LE   320LE

Another ESP                     400 LE

Another Servo motor more stronger

Nema 17 Stepper Motor 17HS6401 60mm High-Torque 2-Phase Hybrid Stepper Motor (1)    940 LE