Server-ESP Communication report                        made by: Abdallah Youssef

Introduction:

In This report we will discuss the server-ESP communication in our project (GREENOVA)

Overview:

the ESP32-CAM microcontroller captures images of the waste inside the bin. These images are transmitted to a Flask web server running on a local computer for processing. The server analyzes the image and sends a response back to the ESP32- with the detected waste type.

Libraries Used:

For the server: we will use Flask which is a lightweight python web framework that creates end point to receive images and send back results

For the ESP:

- WiFi.h: connects the ESP32 to the Wi-Fi network

- HTTPClient.h: sends HTTP POST requests to the Flask server and receives responses.

Sample Code:

ESP Arduino Code:

#include "esp_camera.h"

#include <WiFi.h>

#include <HTTPClient.h>

// ===== WIFI =====

const char* ssid = "AHMED";

const char* password = "KLM1375H";

// ===== NGROK UPLOAD URL =====

// MUST end with /upload

const char* uploadUrl = "https://true-crabs-march.loca.lt/upload";

// ===== CAMERA PINS (ESP32 WROVER KIT) =====

#define PWDN_GPIO_NUM     -1

#define RESET_GPIO_NUM    -1

#define XCLK_GPIO_NUM      21

#define SIOD_GPIO_NUM      26

#define SIOC_GPIO_NUM      27

#define Y9_GPIO_NUM        35

#define Y8_GPIO_NUM        34

#define Y7_GPIO_NUM        39

#define Y6_GPIO_NUM        36

#define Y5_GPIO_NUM        19

#define Y4_GPIO_NUM        18

#define Y3_GPIO_NUM         5

#define Y2_GPIO_NUM         4

#define VSYNC_GPIO_NUM     25

#define HREF_GPIO_NUM      23

#define PCLK_GPIO_NUM      22

void setup() {

Serial.begin(115200);

Serial.println("Booting...");

// ===== CAMERA CONFIG =====

camera_config_t config;

config.ledc_channel = LEDC_CHANNEL_0;

config.ledc_timer = LEDC_TIMER_0;

config.pin_d0 = Y2_GPIO_NUM;

config.pin_d1 = Y3_GPIO_NUM;

config.pin_d2 = Y4_GPIO_NUM;

config.pin_d3 = Y5_GPIO_NUM;

config.pin_d4 = Y6_GPIO_NUM;

config.pin_d5 = Y7_GPIO_NUM;

config.pin_d6 = Y8_GPIO_NUM;

config.pin_d7 = Y9_GPIO_NUM;

config.pin_xclk = XCLK_GPIO_NUM;

config.pin_pclk = PCLK_GPIO_NUM;

config.pin_vsync = VSYNC_GPIO_NUM;

config.pin_href = HREF_GPIO_NUM;

config.pin_sscb_sda = SIOD_GPIO_NUM;

config.pin_sscb_scl = SIOC_GPIO_NUM;

config.pin_pwdn = PWDN_GPIO_NUM;

config.pin_reset = RESET_GPIO_NUM;

config.xclk_freq_hz = 20000000;

config.pixel_format = PIXFORMAT_JPEG;

config.frame_size = FRAMESIZE_QVGA;

config.jpeg_quality = 12;

config.fb_count = 1;

if (esp_camera_init(&config) != ESP_OK) {

Serial.println("❌ Camera init failed!");

return;

}

Serial.println("📷 Camera started.");

// ===== WIFI =====

WiFi.begin(ssid, password);

Serial.print("Connecting to WiFi");

while (WiFi.status() != WL_CONNECTED) {

Serial.print(".");

delay(400);

}

Serial.println("\n✔ WiFi connected");

}

void loop() {

Serial.println("📸 Capturing image...");

camera_fb_t *fb = esp_camera_fb_get();

if (!fb) {

Serial.println("❌ Capture failed");

delay(2000);

return;

}

Serial.printf("Image size: %d bytes\n", fb->len);

// ===== BYPASS SSL HANDSHAKE =====

WiFiClientSecure client;

client.setInsecure();    // <<< THIS BYPASSES CERTIFICATE VALIDATION

HTTPClient http;

http.begin(client, uploadUrl);

// =========================================================

// === AUTHENTICATION HEADER ADDED HERE ====================

// =========================================================

// Base64-encoded value for credentials: :156.202.45.89

const char* authHeaderValue = "Basic OjE1Ni4yMDIuNDUuODk=";

http.addHeader("Authorization", authHeaderValue);

// =========================================================

http.addHeader("Content-Type", "image/jpeg");

// Send POST request

int response = http.POST(fb->buf, fb->len);

Serial.print("Server Response: ");

Serial.println(response);

if (response > 0) {

Serial.println(http.getString());

} else {

Serial.println("❌ Failed to send POST");

}

http.end();

esp_camera_fb_return(fb);

delay(5000); // wait 5 seconds before next photo

}

For tunneling I used local tunneling because its free and work perfectly with ESP32 , other tunneling providers like NGROK Isn’t compatible with ESPs due to conflicts happen in the TLS certification between them

Here is the Local Tunneling:

Flask server code:

This code Establish a flask web server on port 5000 to Receive Pictures sent via POST by the esp 32 Then Saving the pictures to a folder

Output:

After running the Flask server :

Then we enabled tunneling:

Then the Esp will start capturing image and sending it into our webserver

Future steps:

Right now the ESP is configured to send a picture every 1 minute , the next step is to install A PIR Sensor to send a picture only when there is an object placed (the esp will start sending once an object is placed in the trash)